MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

DATE:  February  17,  1994

MS-4402  PCI  VGA  CARD

PERFORMANCE  REPORT

PRODUCT  DESCRIPTION:

Product  Name		:	MS-4402

Chipset				:	Avance  Logic, Inc.  ALG2301

Memory				:	1MB  memory  on  board  (2MB  optional)

Resolution			:	Supports  640 x 480 x 16M  true  colors

Supports  800 x 600 x 64K  colors

Supports  1024 x 768 x 256  colors

Supports  1280 x 1024 x 256  colors  (2MB)

Software  Drivers	:	Version 1.1

CADKey 3.0,  EasyCAD 2.0,  Lotus 1-2-3 R2.x,

PCAD 4.0/4.5,  Framework III,  Symphony 1.x,

VersaCAD v5.4,  WordPerfect  5/5.1, Windows 3.1, 									WordStar 3.3/4.0,  AutoCAD R11/R12, AutoSHADE,

Utility programs such as: ALANSI.SYS, VMODE,

VDIAG and ASCAN

TESTING  CONFIGURATION:

Motherboard		:	MS-5103  PCI  Mainboard

Processor		:	Intel  Pentium  60MHz

Version			:	Ver  2.0

BIOS				:	AWARD  WM63

Chipsets			:	Intel  Mercury

System  Memory	:	16MB  (NEC 424260-80)

Cache  Memory	:	256KB  (MT5C2568-20)

I/O  Controller		:	MS-3805  (NCR  53C810PT)  PCI  SCSI  Controller

VGA  Card			:	MS-4402  (ALI  ALG2301)  PCI  with  1MB  DRAM

Hard  Disk  Drive	:	Maxtor  LXT-340S  HDD  (340 MB)

WINDOWS  TEST  PROGRAMS:

The following programs below were tested using the Windows drivers, under the different supported modes:

| WINTACH 1.2 | 1024x768x256C | 800x600x64K | 640x480x16.7M |
| --- | --- | --- | --- |
| Word Processing | 23.39 | 21.68 | 12.48 |
| CAD/Draw | 107.46 | 64.81 | 36.12 |
| Spreadsheet | 48.38 | 33.81 | 7.08 |
| Paint | 60.56 | 35.12 | 16.44 |
| OVERALL RPM | 59.95 | 38.86 | 18.03 |

| WINBENCH 3.11 | 1024x768x256C | 800x600x64K | 640x480x16.7M |
| --- | --- | --- | --- |
| MS PATCOPY | 43.26 | 15.38 | 1.99 |
| ExtTextOut 16 pt System | 17.05 | 10.83 | 1.18 |
| ExtTextOut 21 pt New Tms Rmn | 8.25 | 4.04 | 1.49 |
| MS PATINVERT | 18.58 | 6.52 | 0.41 |
| POLYGON ALTERNATE FILL | 23.12 | 14.12 | 1.25 |
| Single Horizontal | 41.50 | 19.97 | 1.87 |
| Single Vertical | 5.14 | 4.67 | 1.96 |
| Single Diagonal | 3.49 | 2.35 | 1.36 |
| MS SRCCOPY | 15.37 | 4.22 | 1.79 |
| Poly Mixed | 5.76 | 4.09 | 1.87 |
| WINMARK | 23.51 | 10.02 | 1.66 |

| WINBENCH 4.0 | 1024x768x256C | 800x600x64K | 640x480x16.7M |
| --- | --- | --- | --- |
| Combined GWM1 | 14.7 | 14.4 | 1.82 |
| Combined GWM2 | 15.9 | 5.36 | 0.38 |
| Combined GWM3 | 9.46 | 3.11 | 1.03 |
| Combined GWM4 | 16.6 | 6.31 | 0.64 |
| Combined GWM5 | 16.1 | 6.22 | 0.83 |
| Combined GWM6 | 16.3 | 6.98 | 1.53 |
| Combined GWM7 | 13.4 | 6.42 | 1.60 |
| WINMARK | 14.8 | 6.59 | 1.36 |

| SPEEDY | 81.87 | 33.44 | 5.45 |
| --- | --- | --- | --- |
| WINDSOCK 3.1 | 1006 - SUPERB | 771 - SUPERB | 345 - GREAT |
| WINSTONE 94 | 56.5 | 50.6 | 30.5 |

DOS  TEST  PROGRAMS:

The following programs below were tested under MS-DOS 6.2:

| LANDMARK SPEEDCOM 2.0 |  | 10031 | chr/ms |
| --- | --- | --- | --- |
| QA PLUS 4.61 (DIRECT VIDEO SPEED) |  | 1136085 | cps |
| SUPERSCAPE VGA BENCHMARK V1.0 |  | 58.8 | Frames per sec. |

| DIAGSOFT POWER METER | VER 1.7 | VER 1.81 | Units |
| --- | --- | --- | --- |
| Aggregate Video Test | 6891.8 | 6891.8 | PMUs |
| Character Write Test | 0.6 | 0.6 | Seconds |
| Line Drawing Test | 2.0 | 1.8 | Seconds |
| Area Fill Test | 8.2 | 8.2 | Seconds |

| PC MAG. LABS BENCHMARK | VER 7.0 | Units |
| --- | --- | --- |
| Direct Screen Write (8-bit) | 2195.80 | K char/sec |
| Direct Screen Write (16-bit) | 4809.00 | K char/sec |
| Write Mode 0 Fill (16-bit) | 5243.56 | K pixels/sec |
| Write Mode 0 Fill (32-bit) | 6720.23 | K pixels/sec |
| Mem to Scr BitBlt (16-bit) | 13110.24 | K pixels/sec |
| Mem to Scr BitBlt (32-bit) | 16773.12 | K pixels/sec |
| Scr to Mem BitBlt (16-bit) | 3805.20 | K pixels/sec |
| Scr to Mem BitBlt (32-bit) | 4924.80 | K pixels/sec |
| Scr to Scr BitBlt (16-bit) | 12433.92 | K pixels/sec |
| Scr to Scr BitBlt (32-bit) | 15544.32 | K pixels/sec |
| VIDEO HARMONIC | 6474.97 |  |

| PC MAG. LABS BENCHMARK | VER 8.0 | Units |
| --- | --- | --- |
| Direct Screen Write (8-bit) | 2278.80 | K char/sec |
| Direct Screen Write (16-bit) | 4876.40 | K char/sec |
| Write Mode 0 Fill (16-bit) | 3671.72 | K pixels/sec |
| Write Mode 0 Fill (32-bit) | 5280.40 | K pixels/sec |
| Scr to Mem BitBlt (16-bit) | 3880.32 | K pixels/sec |
| VIDEO HARMONIC | 3845.57 |  |

| DIAGSOFT CHECKIT | VER 3.0 | Units |
| --- | --- | --- |
| BIOS Video Speed | 37647 | Char/S |
| Direct Video Speed | 400543 | Char/S |

| BYTE MAGAZINE BENCHMARK | VER 1.3 | Units |
| --- | --- | --- |
| Text: |  |  |
| Mode 0 | 0.93 | Seconds |
| Mode 1 | 0.97 | Seconds |
| Mode 2 | 0.93 | Seconds |
| Mode 3 | 0.94 | Seconds |
| Graphics: |  |  |
| Mode 4 | 0.22 | Seconds |
| Mode 5 | 0.22 | Seconds |
| Mode 6 | 0.22 | Seconds |
| Mode 13 | 0.77 | Seconds |
| Mode 14 | 0.77 | Seconds |
| Mode 16 | 0.79 | Seconds |
| Mode 18 | 0.77 | Seconds |
| Mode 19 | 0.16 | Seconds |
| VIDEO INDEX | 11.31 |  |

| BYTE MAGAZINE BECNHMARK | VER 2.2 | Units |
| --- | --- | --- |
| Graphics | 2.61 | ips |
| VIDEO INDEX | 4.46 |  |