MSCMOUSE Version 8.00

For Mouse Systems' 2- and 3-button input devices.

Copyright (C) 1984-1992  Mouse Systems Corporation.

INSTALLATION NOTE:

If you already have a copy of MSCMOUSE.COM and/or MSCMOUSE.SYS, just copy

the files accompanying this diskette into the same subdirectory where your

copies already exist.  Then reboot your computer to activate the version

8.00.

VALID COMMAND LINE OPTIONS:

/!ccc - Button assignment.  c can be L, M, or R.

/1    - Use serial port COM1.

/2    - Use serial port COM2.

/A0   - Turn off mouse acceleration.

/A#   - Set sensitivity of dynamic accelerator (# = 1 - 9).

/C    - Use Bus Card.

/Dnnn - Double speed threshold.  nnn can be from 0 to 100.

/E    - Enable/Disable resolution doubling (for 3 button PC Mouse).

/Hnnn - Horizontal sensitivity.  nnn can be from 0 to 100.

/I    - Ignore/Listen to application's acceleration commands.

/Np,i - Use hexadecimal I/O port (p) and IRQ level (i).

/P    - Use PS2-compatible Mouse on Pointing Device Port.

/Q    - Suppress driver screen text.

/R    - Remove COM port from system configuration (for BASIC programs).

/Snnn - Horizontal and vertical sensitivity.  nnn can be from 0 to 100.

/Vnnn - Vertical sensitivity.  nnn can be from 0 to 100.

/U    - Unload mouse and pop-up driver and release memory.

/?    - The help display.

The command line options are used on the same line as the MSCMOUSE command.

The usage is as follows:

MSCMOUSE /C/A5/E/I

or

DEVICE=\MOUSE\MSCMOUSE.SYS /C/A5/E/I

MICROSOFT WINDOWS

-----------------------------------------------------------------------------

This diskette contains another document file, WINDOWS.DOC.  This text file

includes instructions for the installation of your Mouse Systems mouse with

Microsoft Windows version 3.0 and 3.1.