Installation Guide

Reference Manual

for

Universal OS/2 Device Driver

for

DigiCHANNEL Intelligent

Communications Boards

92000100C

DigiBoard, DigiCHANNEL, PC/Xi, PC/16i, PC/8i, PC/Xe, PC/16e, PC/8e, PC/4e,

PC/2e, Xem, PC/Xem, MC/Xem, EISA/Xem, COM/Xi, COM/8i, COM/4i, MC/Xi, MC/16i,

MC/8i, MC/4i and C/X are trademarks of DigiBoard.  All other brand and product

names are the trademarks of their respective holders.

c Digi International, Inc.

All Rights Reserved

Information in this document is subject to change without notice and does not

represent a commitment on the part of DigiBoard.

DigiBoard provides this document "as is," without warranty of any kind, either

expressed or implied, including, but not limited to, the particular purpose.

DigiBoard may make improvements and/or changes in this manual or in the product(

s) and/or the program(s) described in this manual at any time.

This product could include technical inaccuracies or typographical errors.

Changes are periodically made to the information herein; these changes may be

incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used, disclosed,

and/or copied only as permitted under such license agreement.  Any copy must

contain the above copyright notice and this restricted rights notice.  Use,

coping , and/or disclosure of the programs is strictly prohibited unless

otherwise provided in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to restrictions

as set forth in sub-paragraph (c)(1)(ii) of the Rights in Technical Data and

Computer Software clause of DFARS 52.227-7013.

Table of Contents

Table of Contents	3

Introduction	4

The Distribution Diskette	6

Installing the Device Driver	7

Global Parameters	7

Per-Card Parameters	10

Examples	13

Testing the Ports	15

Troubleshooting	16

Error Messages	18

DMODE I/O Port Configuration Utility	19

Examples	23

Appendix-Synchronous Communications Modes	25

Introduction

This release of the DigiWARE device driver for OS/2 supports the

entire line of DigiCHANNEL intelligent asynchronous serial

communications controllers:

Micro Channel (MCA) bus:

DigiCHANNEL MC/Xi 4, 8 and 16 port boards

DigiCHANNEL MC C/X cluster controller system

DigiCHANNEL MC/Xem

PC/AT (ISA) bus:

DigiCHANNEL PC/Xe 2, 4, 8 and 16 port boards

DigiCHANNEL PC/Xi 8 and 16 port boards

DigiCHANNEL COM/Xi 4 and 8 port boards

DigiCHANNEL ISA C/X cluster controller system

DigiCHANNEL PC/Xem

EISA bus:

DigiCHANNEL EISA C/X cluster controller system

DigiCHANNEL EISA/Xem

The device driver software supports multiple boards and multiple

board types in a single machine, and can handle up to 256 ports

from any combination of the boards listed above.  As new products

are introduced, the driver will be upgraded to include those devices

as well.

The driver takes all its configuration information from the device=

command line in CONFIG.SYS, and all DigiCHANNEL intelligent

boards in a system are configured from a single command line.

In addition to the device driver, the configuration utility

DMODE.EXE is provided to change communication parameters for

individual ports without rebooting.  DMODE.EXE is similar to the

OS/2 MODE command, but has some added features, and supports

communications devices beyond OS/2's range of COM1 to

COM8.

Before installing this device driver software,

complete the installation of the DigiCHANNEL

intelligent communications board(s) in your

system, according to the hardware installation

manual provided with the board(s), and write

down your configuration!  The driver supports all

legal I/O and memory address settings, but must

be informed of these settings in the device driver

command line.

The Distribution Diskette

The diskette accompanying this manual contains all of the files and

documentation needed to install the DigiWARE OS/2 Universal

Intelligent Device Driver in your system.  A list of these files

follows:

XALL.SYS		The installable device driver.

DMODE.EXE		A MODE work-alike program for configuring I/O

ports beyond COM8.

INSTALL.DOC		Installation instructions for the current

release of the driver.

RELNOTES.DOC		Release notes.

DMODE.DOC		Instructions for the DMODE program.

XALL.DOC		Programmer's information about the driver

and applicable system calls.

*.bin			Miscellaneous binary files used by the driver.

Installing the Device Driver

Copy all of the files from the distribution diskette to a single

directory on your hard drive.  By default, driver and data files are

expected to be in \DIGI.

The device driver is invoked by placing its name and configuration

arguments into the system's CONFIG.SYS file.  All Digi-

CHANNEL intelligent asynchronous serial communications boards

must be installed from a single command line, which contains

complete set-up information for each board.  The format of the

command line is:

device=\digi\xall.sys [/n:names] [/b:baud] [/d:dtype]

[/f:digipath] [/v:messagepath] /t:card /p:port /m:mem [,

/t:card /p:port /m:mem] [, ...]

Note that the path for XALL.SYS is \DIGI.  This is the default

path.  If an alternative path is specified with the /f:digipath option,

substitute that path for \DIGI.

The above command line must appear on a single

command line in the CONFIG.SYS file.  This line

may be up to 255 characters long.

Global Parameters

The first six parameters in the command line above are global

parameters, and affect all boards installed.  These parameters are

optional, and may be omitted.  If omitted, default values will be

used.

/n:names

Defines the naming convention to be use for the

DigiCHANNEL ports.  names consists of an optional

"stem" followed by the starting device number.  The

new stem must be enclosed in double-quotes ("").  If

the stem is omitted, "COM" will be used.  If the

number is omitted, "3" will be assumed.  If the /n

parameter is not specified, the default name COM3 will

be assigned to the first port.

/n:"digi"0	DIGI0, DIGI1, DIGI2, ... DIGIn

/n:4			COM4, COM5, COM6, ... COMn

/n:"qdev"	QDEV3, QDEV4, QDEV5, ... QDEVn

/b:baud

Sets the default baud rate to be used for all ports.  If this

parameter is not specified, all ports will be set to the

OS/2 default of 1200 baud.  Baud rates of up to 38,400

are supported.  Use the DMODE.EXE command to set

baud rates of individual ports.

/b:9600		Initializes all ports to 9600 baud.

/d:dtype

Sets the initial data type (parity, character length and

number of stop bits).  If this parameter is omitted, the

OS/2 default data type of even parity, 7 data bits and 1

stop bit is used.  The syntax is pds, where p is E, O or N;

d is 8, 7, 6 or 5; and s is 1 or 2.  All three values must

be supplied, in the correct order.

/d:N81		No parity, 8 data bits, 1 stop bit.

/f:digipath

Defines an alternative directory path for the driver and

data files.  If this option is present, all files from the

distribution diskette must be present in this directory.

/f:d:\drvr\digi

All data files are in "D:\DRVR\DIGI".

/v:messagefile

Forces the driver to direct all verbose messages to the

file named.  If /v: is given without a filename,

messages will be placed in \XALL.MSG.  This flag

prevents any information from being displayed on the

console; useful in some situations as when the machine

is a "headless server", where calls to Standard Output

will cause problems.  Without this flag, all messages will

appear on the system console.

The message file is overwritten each time the system is

started.

/v:errmsg 	Writes banner and other info to file

"\ERRMSG".

/v:			Writes banner and other info to file

"\XALL.MSG".

/a:a		Enable the alternate pin assignment of Data Carrier

Detect (DCD) for all ports (see discussion of Alt-Pin

processing on page 24).

Per-Card Parameters

The remaining three parameters are per-card parameters, and are

used to inform the driver of the type of board and where it is to be

addressed in the host's I/O and memory buses.  These parameters

are mandatory, and must be given for each board!

The per-card parameters must be given in complete sets, and the

sets must be separated by commas.

/t:card

Defines the card type to the driver.  Legal values for

card are:  i for the ISA C/X, e for the EISA C/X, m for

the MC C/X, p for Xem systems (PC/Xem, MC/Xem

or EISA/Xem) and x for COM/Xi boards.  This

parameter must be omitted for PC/Xe, PC/Xi and

MC/Xi boards.  The designators i, e, and m must be

followed by an additional two digits defining the

number and arrangement of DigiCHANNEL C/CON-

16 or C/CON-16e concentrators on each host adapter

channel.  If the concentrator(s) are connected to the host

adapter via a dial-up or leased line (remote

concentrators), each of the two digits must be followed

by a parameter, enclosed in parentheses, specifying the

synchronous communications mode to be used for that

line.  See the Appendix for a list of available

synchronous communications modes.

/t:i10	ISA C/X, one local concentrator on line 1

only.

/t:e11	EISA C/X, one local concentrator on each of

lines 1 and 2.

/t:m21	MC C/X, two local concentrators on line 1

and 1 on line 2.

/t:e12	EISA C/X, one local concentrator on line 1,

and two on line 2.

/t:i22	ISA C/X, two local concentrators on each of

lines 1 and 2.

/t:i2(21)3(27)

ISA C/X, two remote concentrators on line

1, and two remote concentrators on line 2.

The concentrators on line 1 will use mode

#21 (76,800 baud, 8-wire, external clock) to

communicate with the host adapter, and the

concentrators on line 2 will use mode #27

(614,400 baud, 8-wire, internal clock).  See

the Appendix for a list of synchronous

communications modes.

/t:x		COM/Xi (4 or 8 channels)

/t:p		PC/Xem, MC/Xem or EISA Xem

/p:port		Defines the I/O port address for the card.

/p:300		Board is addressed at I/O port 300h.

/p:2005		EISA board (EISA C/X or EISA/Xem

host adapter) is in slot 2 (EISA I/O

addresses are 4 digits-the slot number,

followed by "005").

When plugging an ISA board (PC/Xe, ISA C/X host

adapter, etc.) into an EISA machine, the /p parameter

must contain the 3-digit I/O address as set by the DIP

switches on the board, and not the 4-digit address based

on the EISA slot number.

/m:mem

Dual-ported memory starting address.  This is the full

32-bit hexadecimal address of the beginning of the

board's dual-ported memory.

/m:d0000	Sets dual-ported memory start

address to 0D0000h (in the BIOS

expansion area between 640K and 1

megabyte).

/m:e80000	Sets start address of E80000h (in the

fifteenth megabyte).

EISA and ISA boards (except the COM/Xi), may all

share the same memory starting address.  COM/Xi and

all Micro Channel boards must each have unique

memory addresses.

Examples

The following examples start out with just the minimum to install a

DigiCHANNEL board.  After locating an example for your type of

board, read the rest of the examples for optional situations that may be

appropriate to your application.  Default options will be altered by your

application program(s) when you configure the application.

Example 1:	COM/Xi

One COM/Xi at I/O port 300h, memory start address D8000h;

devices to be named COM5 COM6, etc.  All data files are in the

directory \DIGI:

device=c:\digi\xall.sys /n:5 /p:300 /m:d8000 /t:x

(Note that the above must all appear on one line in CONFIG.SYS)

Example 2:	PC/Xe, PC/Xi

One PC/Xe (or PC/Xi) at I/O port 220h, memory address D0000h

Ports will be named COM3, COM4, etc. (default names).  All data

files are in the directory \MISC\DIGI:

device=c:\misc\digi\xall.sys /f:\misc\digi /p:220 /m:d0000

(Note that the above must all appear on one line in CONFIG.SYS)

Example 3:	MC/Xi

One MC/Xi at I/O port F1F0h, memory address FC0000h.  Devices

will be named DIGI1, DIGI2, etc. and will have an initial baud rate of

38,400.  All data files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"digi"1 /b:38400 /p:F1F0 /m:FC0000

(Note that the above must all appear on one line in CONFIG.SYS)

Example 4:	Two board installation with optional global

parameters.

Two PC/Xi (or PC/Xe) boards at I/O ports 320h and 300h, with dual-

ported memory of both starting at D0000h.  Ports will be named

COM0, COM1, etc., and will be initialized to 38,400 baud, no parity,

8 data bits and 1 stop bit; messages will be contained in the file

MESSAGES.MSG located in the \DIGI directory.  All data files are in

the default directory \DIGI:

device=c:\digi\xall.sys /n:0 /b:38400 /v:\digi\messages.msg

/d:N81 /p:320 /m:D0000, /p:300 /m:D0000

(Note that the above must all appear on one line in CONFIG.SYS)

Example 5:	PC/Xem

One PC/Xem at I/O port 224h, memory start address C8000h; devices

to be named COM3 COM4, etc.  All data files are in the default

directory \DIGI:

device=c:\digi\xall.sys /p:224 /m:C8000 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

Example 6:	ISA C/X

One ISA C/X host adapter at I/O port 228h, memory address

D0000h, with one concentrator on line 1 and two on line 2.  Devices to

be named DCX1, DCX2, etc. and will have an initial baud rate of

38,400.  All data files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"dcx"1 /b:38400 /p:228 /m:d0000 /t:i12

(Note that the above must all appear on one line in CONFIG.SYS)

Example 7:	EISA C/X with a remote concentrator via an RS-

232 synchronous modem.

One EISA C/X host adapter in slot 3, memory address D8000h, with

two local concentrators on line 1 and one remote concentrator at 9600

baud and external clocking on line 2.  Devices to be named COM5,

COM6, etc. and will have an initial baud rate of 19,200.  All data files

are in the directory \OS2\DIGI:

device=c:\os2\digi\xall.sys /n:5 /b:19200 /f:c:\os2\digi

/p:3005 /m:d8000 /t:e21(17)

(Note that the above must all appear on one line in CONFIG.SYS)

Example 8:	MC C/X and MC/Xem

One MC C/X host adapter at I/O port 228h, memory address

D8000h, with one concentrator on line 1 and one concentrator  on line

2; one MC/Xem at I/O Port 304h, memory address D0000h.  Devices

will be named DIGI1, DIGI2, etc. and will have an initial baud rate of

38,400, Alt-Pin enabled, no parity, 8 data bits, and 1 stop bit.  All data

files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"digi"1 /b:38400 /a:a /d:N81 /p:228

/m:d8000 /t:m11, /p:304 /m:d0000 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

(Note that for MC C/X and MC/Xem, the addresses 104 and 108

should not be used to avoid possible conflicts)

Testing the Ports

The simplest (and probably the best) way to check the driver and

I/O connections is to connect an ASCII terminal to one of the ports

and redirect some output to that port.  If the terminal is connected to

COM3, try something like the following:

dmode com3	(this will verify that the driver can find the

port-the port's parameters will be displayed.)

dmode com3 octs=off odsr=off

(this insures that the hardware handshaking

signals CTS and DSR will not prevent output-

we are not concerned with flow control at this

time.)

dir > com3	(this redirects the current directory to COM3.)

The console should simply respond by repeating the C:\> prompt,

and the current directory (in this case, C:\) will appear on the

terminal.

Troubleshooting

If the C:\> prompt doesn't reappear, and the system seems to be

"hung", you probably used a memory start address that is being

used by some other device, or the board is not set for the address

specified in CONFIG.SYS.  Try a different address.  If it still hangs,

try a different I/O port.  If your computer has a memory cache and

the board's memory start address is above 1 megabyte, try either

disabling the cache or moving the board to a low-memory address

(D0000 is usually a good choice).

It is also possible that a conflict exists with another device driver

which has been loaded via CONFIG.SYS.  The XALL.SYS line

should be placed near the end of the CONFIG.SYS, and must not

be placed before the DEVICE=TESTCFG.SYS line.

Due to potential device naming conflicts, the pair of lines

DEVICE=\OS2\COM.SYS

and

DEVICE=\OS2\VCOM.SYS

may cause problems.  Be sure that the XALL.SYS line is placed

before these lines in CONFIG.SYS.  If the XALL.SYS driver

generates an error message, try temporarily putting "REM" in front

of the above two lines.  If this works, there is probably a naming

conflict.  You may need to add a /n: command line flag to the

XALL.SYS line to force the driver to use a different base name or

start numbering the COM devices at 5 or above.

If the C:\> prompt does reappear, but there is no output on the

terminal screen, there is probably a cabling problem.  Make sure

that all cable plugs are securely seated in receptacles, and that they

are configured correctly (see the appropriate section of the

hardware installation guide for your board).

If the terminal displays output, but the data on the screen is garbled,

the terminal is probably not set with the same baud rate and data

type as specified in CONFIG.SYS (or the defaults, if not specified).

Verify these settings.

If everything checks out and you still have problems, there could

be a problem with your board.  Contact your distributor or

DigiBoard Technical Support for further assistance.

Error Messages

If, when the driver is loaded, it cannot find the board(s), or if they

fail to respond properly to commands, certain error messages may

be sent to the console.  These error messages are defined in the

following table:

Could not find BIOS file

Could not find FEP file

Could not find data file

One or more files were missing from the digipath directory.

The default digipath directory is /digi.  If you have loaded the

driver and data files into a different directory, that directory

must be specified in the xall command line with the

/f:digipath parameter.

IO Port Not Responding

I/O port address set incorrectly, or conflicting with some other

device. Verify setting, or try a different address.

DigiBoard BIOS Will Not Boot

DigiBoard BIOS Will Not Respond

DigiBoard FEPOS Will Not Boot

BIOS on DigiCHANNEL board will not respond to commands

from the driver.  Typically due to a memory conflict, or a

memory cache conflict.

Driver Failed to Hook Timer

System Cannot Allocate Memory

A system call to OS/2 has failed.

Invalid Command Line Arguments

Syntax error in the command line in CONFIG.SYS.  Double-

check your entries.

DMODE I/O Port Configuration Utility

DMODE is DigiBoard's I/O channel configuration utility. DMODE

functions like the OS/2 MODE command, but is more flexible, and

provides some added features.

*	Supports communications device names other than COM1-

COM8.

*	Any single parameter may be changed without causing all

others to be reset to default values.

*	Can be used to enable or disable the Alt-Pin feature on

individual ports.

*	Maintains the familiar MODE syntax.

DMODE arguments may be entered in any order, and in many cases

may be abbreviated.  For example, a baud rate of 19,200 may be

entered simply as "19", since within the context of the DMODE

command, "19" cannot be interpreted as anything else. Arguments

may be separated by spaces or commas, and unspecified

parameters are left unchanged.  DMODE returns the settings of all

parameters after any requested changes have been made. If no

parameters are given, DMODE simply returns the current settings.

The syntax of DMODE is:

dmode port_name [parameters]

Port names and parameters are not case sensitive, so upper and

lower case letters may be freely mixed without effect on the

operation of the DMODE command.

The following parameters are accepted:

Baud Rate	Baud rates can be specified with 2 to 5 digit

designations, according to the following table:

Accepted Values            Baud Rate

11   110                   110

15   150                   150

30   300                   300

60   600                   600

12   120   1200            1200

24   240   2400            2400

48   480   4800            4800

96   960   9600            9600

19   192   1920   19200    19200

38   384   3840   38400    38400

56   567   5670   56700    56700

Character Length

Characters may be 5, 6, 7 or 8 bits long.  Enter the

appropriate number in the DMODE arguments.

Parity		Enter the desired parity checking method with a single

letter E, O, N, M or S for Even, Odd, None, Mark or

Space, respectively.

Stop Bits	Enter the desired number of stop bits.  Recognized

values are 1, 2 and 1.5.

Flags		DMODE supports the following flags in the same way as

the OS/2 MODE command.  Refer to your OS/2

documentation for explanations of the flags. To set a

flag, enter the mnemonic for the flag, followed by an

equals sign (=), and the desired value (ON, OFF, etc.).

Read Timeout Flag

TO=ON		Read timeout in effect.

TO=OFF	No timeout.

XON/XOFF Handshake Flag

XON=ON	XON/XOFF handshaking enabled.

XON=OFF	XON/XOFF handshaking disabled.

Input DSR Sensitivity

IDSR=ON	Enables Input DSR Sensitivity.

IDSR=OFF	Disables Input DSR Sensitivity.

Output DSR Sensitivity

ODSR=ON	Enables Output DSR Sensitivity.

ODSR=OFF	Disables Output DSR Sensitivity.

Output Handshake on CTS

OCTS=ON	Enables CTS output flow control.

OCTS=OFF	Disables CTS output flow control.

DTR Signal Control

DTR=ON	Sets DTR to "ON" or HIGH.

DTR=OFF	Sets DTR to "OFF" or LOW.

DTR=HS	DTR used for output flow control.

RTS Signal Control

RTS=ON	Sets RTS to "ON" or HIGH.

RTS=OFF	Sets RTS to "OFF" or LOW.

RTS=HS	RTS used for output flow control.

RTS=TOG	Toggles RTS from LOW to HIGH, or

from HIGH to LOW.

Alt-Pin Processing

ALT=ON	Internally swaps DSR and DCD signals

(see explanation below).

ALT=OFF	DSR and DCD are not swapped.

DigiCHANNEL products use 10-pin RJ-45

connectors in order to support all of the signals

normally available on a 9-wire RS-232 connection.

However, many commercially available RJ-45

connectors have only 8 pins.  8-pin RJ-45 plugs will

fit in a 10-pin jack, but the first and last pins

(numbers 1 and 10) are not connected.  DigiBoard

uses these pins for the modem control signals RI

(Ring Indicator, pin 1) and DCD (Data Carrier

Detect, pin 10). While Ring Indicator is rarely a

concern (since most modems have auto-answer

capability), Data Carrier Detect is often monitored to

insure that the modem has established, and is

maintaining, a valid connection with a remote

modem.

To facilitate the use of 8-pin RJ-45 connectors in

modem applications, a special DMODE flag, ALT, has

been provided.  ALT instructs the device driver to

monitor pin 1 of an 8-pin RJ-45 for DCD.

See the Hardware Installation guide for your board

for cabling diagrams and pin assignment charts for

DB-25 and RJ-45 connectors.

Examples

dmode digi12

Returns the current settings of device digi12.  No changes are

made.

dmode com5 2

Configures COM5 for 2 stop bits.  All other parameters remain

unaffected.

dmode com5:38 rts=on

Configures COM5 for a baud rate of 38,400 and sets RTS to "ON"

or HIGH.

dmode com5 8

Configures COM5 for 8 data bits.  No other parameters are

affected.

dmode comdv83 idsr=off, to=off e 7 12

Sets COMDV83 to disable Input DSR Sensitivity, disable input

timeout, select even parity, 7 data bits and a baud rate of 1200.

dmode com5 38 alt=on

Sets up COM5 for a baud rate of 38,400, and enables Alt-Pin

Processing.

dmode com5 e alt=off

Sets COM5 for even parity checking and disables Alt-Pin

processing.

dmode com3 odsr=off octs=off

Disables sensing of output flow control signals Clear To Send

(CTS) and Data Set Ready (DSR).

Appendix-Synchronous Communications Modes

The DigiCHANNEL C/X host adapter communicates with attached

C/CON-16 concentrators via two RS-422 synchronous channels

(newer concentrators and host adapters also support RS-232

synchronous communication-see your hardware Installation

Guide).  The default operating mode for these channels is 1.2

megabaud, 8-wire, internally clocked.  This provides the fastest

possible communication between the host adapter and locally

connected concentrators (in this context, local indicates that

concentrators are connected directly, via a 4 or 8 wire cable, to the

host adapter; remote refers to concentrators that are connected to

the host adapter via data communications devices such as modems

or DSUs).

Local concentrators always communicate with the host adapter in

either an eight wire internally clocked mode or a four wire self-

clocked mode (in the four wire modes the clock is encoded with

the data instead of being carried on separate lines as in the eight

wire modes).

Remote concentrators always use an eight wire externally clocked

mode (the clock signal is provided by the synchronous modem).

The table on the following page shows the modes that are

supported by the XALL.SYS device driver.  Note that the external

clock baud rates are approximates used to set up time-outs and

tuning parameters only.  Values do not need to be exact; select the

speed closest to the actual sped of your synchronous modem.

Eight-Wire               Eight-Wire              Four-Wire

Internal Clock           Enternal Clock          Self-Clocking

Mode   Baud Rate         Mode   Baud Rate        Mode   Baud Rate

Mode

0      115k              15     2400             1      250k

16     4800             2      460k

3      2400              17     9600

4      4800              18     19.2k

5      9600              19     38.4k

6      19.2k             20     57.6k

7      38.4k             21     76.8k

8      57.6k             22     115k

9      76.8k             23     230k

10     115k              24     460k

11     230k              25     920k

12     460k              26     1.2M

13     920k

14     1.2M              29     614k

30     737k

27     614k

28     737k