AIOESPCA.NLM Version 2.00

Hayes Microcomputer Products, Inc.

P.O. Box 105203

Atlanta, GA. 30348

TEL: (404) 441-1617      BBS: (800) 874-2937 (800) US-HAYES

FAX: (404) 449-0087      BBS: (404) 446-6336 (404) HI-MODEM

Copyright (C) 1993 Hayes Microcomputer Products, Inc

All rights reserved.

(September 1, 1993)

This driver allows AIO applications to use the serial ports

on a Hayes ESP Communications Accelerator installed in the server.

Each board requires its own interrupt, which is allocated to the

first port, or Master Port.  Additional ports (Slave Ports) share

the interrupt, unless specifically assigned an exclusive interrupt

using the "slavex=" parameter.

Interrupts (hex) 3, 4, 5, 7, 9, A, B, C, E, and F are supported.

As many boards may be installed as interrupt availability allows,

up to a maximum of ten boards.

The operator must issue one LOAD command per board.  LOADing is

typically performed by the AUTOEXEC.NCF file at startup time.

To load the AIOESPCA NLM use:

"load aioespca port=xxx int=x [slavex=yyy] [node=x] [byteio]

[name=x] [prescalex=y]"

The brackets indicate optional parameters.

REQUIRED load parameters (prompted for if not on command line)

port       Specifies I/O address (in hex) used by ESPCA board.

This must match the jumpered I/O address designation for

the intended board.  Several hardware-jumpered I/O

Addresses are available, with the remaining addresses

available via software configuration of the board's

nonvolatile memory.

Valid choices are: 100, 140, 180, 200, 240, 280, 300, 380.

int        Specifies interrupt used by the first port, or Master

Port.  Other ports share this interrupt unless

specifically assigned an exclusive interrupt using the

"slavex=" parameter.

Valid choices are: 3, 4, 5, 7, 9, A, B, C, E, F.

OPTIONAL load parameters:

slavex     Specifies interrupt used by a particular slave port.

Slaves are numbered starting with 1, with the number

replacing the "x" in "slavex".  For example:

slave1=7   (assign interrupt 7 to first Slave Port)

Valid choices are: 3, 4, 5, 7, 9, A, B, C, E, F.

node       Specifies board number used by ESPCA adapter. If this

parameter is not specified, the driver will assign one

based on I/0 address.

byteio     By default, the AIOESPCA driver performs 16-bit I/O

data move operations, which require a 16-bit bus slot.

The AIOESPCA driver and ESPCA board may operate in an

8-bit slot by specifying this switch.

Valid choices are: 0-9.

name       The user-defined name which correlates to the board

loaded.

prescalex  Specifies a baud rate multiplier for a particular port.

Baud rates can be doubled, quadrupled, or octupled using

this switch.  For example:

prescale0=1  (doubles Master Port baud rates)

prescale1=3  (octuples first Slave Port baud rates)

Valid choices are: 0-3

0 - x1

1 - x2

2 - x4

3 - x8