AHA-1540/42B INSTALLATION GUIDE

HARDWARE INSTALLATION

The Adaptec AHA-154XB AT-to-SCSI Host Adapter has been designed to operate

as shipped in the majority of AT class computers.  The Host adapter's

factory default settings should remain in their original positions.  The

AHA-154XB is shipped with the following settings:

SCSI Address                    7

SCSI Parity                     Enabled

Terminators                     Installed

Terminator Power                Supplying

Synchronous Negot.              Disabled

DMA Channel                     5

Interrupt Channel               11

AT Port Address                 330h

AT Bios Address                 DC000H, Enabled

Bios Wait State                 0, Disabled

FD Controller                   Enabled (AHA-1542B)

DMA Transfer Speed              5 MB/second

Auto Sense                      Enable

TO PERFORM INSTALLATION:

TURN OFF POWER TO THE SYSTEM AND EXTERNAL EQUIPMENT.

Remove the cover of your AT personal computer to expose the AT bus slots.

Locate an unused AT expansion slot in your AT system.  AT type slots can be

recognized by the fact that they have 2 edge connectors, one 62-pin and the

other 36 pin.

Remove the corresponding system expansion slot cover by turning the screw

that secures it from the top counter-clockwise.

Align the AT I/O Bus Connector on the bottom of the AHA-154X into this AT

slot.  Use the screw from the corresponding expansion slot cover to secure

the AHA-154X's bracket to your AT system frame.

The SCSI bus must also be terminated correctly to ensure proper operation.

The first and last physical SCSI devices on the SCSI cable must have

terminators installed.  All other SCSI devices must have terminators

removed.

The SCSI factory-installed Bus terminators on the AHA-154X should be removed

if and only if you attach SCSI devices to both an internal and external cable

since the internal and external cable SCSI connectors are the same SCSI Bus.

Attach the SCSI Bus to the host adapter and the peripherals, using either

the internal or external connector, making sure that pin 1 orientation is

maintained throughout the bus.

If the host adapter is not the first or the last unit on the cable, remove

the SCSI terminators from the board.

Reassemble the system in the reverse order.

JUMPER CONFIGURATION REFERENCE

Five sets of jumpers are located on the board to configure user-selectable

options.  Jumpers installed at the factory are shown as "(x)".  Those not

installed are shown as "o".  It should not be necessary to change the

jumper settings.

J5-GENERAL CONTROL

Pin  1  2  3  4  5  6  7  8  9  10  11  12  13

o  o  o  o  o  o  o (x) o  (x)  o   o   o

o  1 - Synchronous Transfer negotiation enable.

o  2 - Diagnostics (used only at Adaptec).

o  3 - SCSI Parity disable.

o  4  o   (x)   o   (x)   o   (x)   o   (x)  - SCSI Address ID

o  5  o    o   (x)  (x)   o    o   (x)  (x)    (SCSI disks should be

o  6  o    o    o    o   (x)  (x)  (x)  (x)    set for ID 0 and 1)

----------------------------------------

7    6    5    4    3    2    1    0

o  7  o  (x)   o   (x) - DMA Channel Select

x  8  o   o   (x)  (x)   See also jumper J9

-------------------

7   6    5    0

o  9  o   (x)   o   (x)   o   (x) - Interrupt Channel Select.

x 10  o    o   (x)  (x)   o    o    See also jumper J9

o 11  o    o    o    o   (x)  (x)

---------------------------

9   10   11   12   14   15

o 12   o   (x)   o   (x) - DMA Transfer Speed

o 13   o    o   (x)  (x)   in MB/sec.

------------------

5.0  5.7  6.7  8.0

J6-BIOS/Auto Sense Control

Pin  1  2  3  4  5

(x) o  o  o  o

1 - BIOS Enable.

2,3,4 - Not Used.

5 - Auto Sense disable

J7-ADDRESS SELECTION

Pin  1  2  3  4  5  6  7  8

o (x) o  o  o  o  o  o

o  1 - Floppy Secondary Address select (AHA-1542B only).

x  2   o   (x)   o   (x)   o   (x) -  AT I/O Port Address Select

o  3   o    o   (x)  (x)   o    o    in Hexadecimal

o  4   o    o    o    o   (x)  (x)

----------------------------

334  330  234  230  134  130

(Note: The host adapter bios is "hard coded" to 330H)

o 5   o   (x)   o   (x) - BIOS Wait State Select

o 6   o    o   (x)  (x)   in nanoseconds

------------------

0   100  200  300

o 7   o    (x)    o    (x)   - BIOS Base Address Select

o 8   o     o    (x)   (x)   in Hexadecimal

-----------------------

DC000 CC000 D8000 C8000

J8-FLOPPY DISK SELECTION (AHA-1542B only)

Pin  1   2   3   4   5   6   7  8

(x) (x)  o  (x)  o  (x)  o  o

1 - Floppy enable

2,3 - DMA Request 2 or 3 select

4,5 - DMA ACK 2 or 3 select

6,7 - INT Request 6 or 10 select

8 - Dual Speed enable

J9-DMA/INTERRUPT SELECTION

Pin  1  2  3  4  5  6  7  8  9  10  11  12  13  14

o (x) o  o  o (x) o  o  o   o  (x)  o   o   o

o  1 - DMA Request 0 Select

x  2 - DMA Request 5 Select

o  3 - DMA Request 6 Select

o  4 - DMA Request 7 Select

o  5 - DMA ACK 0 Select

x  6 - DMA ACK 5 Select

o  7 - DMA ACK 6 Select

o  8 - DMA ACK 7 Select

o  9 - INT Request 9 Select

o 10 - INT Request 10 Select

x 11 - INT Request 11 Select

o 12 - INT Request 12 Select

o 13 - INT Request 14 Select

o 14 - INT Request 15 Select

SOFTWARE INSTALLLATION

Copy the files from the ASW-1410 Program diskette to the root directory of

drive C using the MS-DOS COPY command.  In order for the ASPI MS-DOS Manager

files to be loaded whenever the system is booted, the CONFIG.SYS file must

contain the following DEVICE=commands:

DEVICE = ASPI4DOS.SYS

DEVICE = ASPIDISK.SYS

Using any ASCII file editor, add the individual device driver commands to an

existing CONFIG.SYS file, or follow the Instructions in the MS-DOS Operations

Reference Manual to create a new CONFIG.SYS file.  FAILURE TO LOAD ALL THE

DEVICE DRIVERS IN THE CORRECT ORDER, CAUSES THE PROGRAM TO NOT BE INSTALLED.

The following command line options may be added to the device drivers in the

CONFIG.SYS file.  (Refer to the User's Manual for details).

DEVICE = ASPI4DOS.SYS (/D) (/P<port address) (/L) (/V) (/W) (/X<speed>)

(/N<bus on time>) (/F<bus off time>) (/I).

MS-DOS provides access to physical hard disk drives C and D.  You must use

the MS-DOS FDISK and FORMAT programs for drives C and D.  DO NOT use AFDISK

with drive C and D.  To use a hard disk partitioned by AFDISK as drive C or

D, you must use AFDISK to delete the partitions, and then use MS-DOS FDISK

to create new partitions.

When starting AFDISK program, it must be run from the root directory of

drive C or the AFDISK directory if an AFDISK directory was created.  When

ready, type:

AFDISK (at the DOS prompt)

and press ENTER.  The AFDISK utility is menu driven.  Follow the directions

on the screen to partition and format the selected SCSI devices.  The AFDISK

utility also includes an on-line HELP file which is accessed by pressing the

F1 key.  The AFDISK may be exited at any time without executing by pressing

the ESC key and being careful to choose the selections that do not write to

or format the disk.

When the SCSI device(s) are successfully partitioned and formatted, reboot

the system.  This saves any changes made and allows use of the newly

installed disk(s) or partition(s).

TERMINATION

The SCSI bus must be terminated correctly to ensure proper operation.  The

first and last physical SCSI devices on the SCSI cable must have terminators

installed.  All other SCSI devices must have terminators removed.  The AHA-

154XB host adapter is usually the first device on the SCSI Bus and has

terminators installed at the factory.

(x)  - SCSI Address ID

o  5  o    o   (x)  (x)   o    o