Packet Drivers, made simple             9 Jan 1990

by Joe R. Doupnik, Utah State University

jrd@usu.Bitnet, jrd@cc.usu.edu 129.123.1.1

for Novell Technology Transfer Partnership

The problem:

Ethernet is a common, or the most common, large organization method

of networking systems. To date, the many programs (applications) designed to

talk across Ethernet all have built-in drivers for particular Ethernet boards.

That means that each application owns the Ethernet board and other

applications cannot use it simultaneously. Goodbye Novell file server(s) if

TCP/IP is needed.

That also has meant that different applications have not used a common

pool of Ethernet boards and the user may have to install two of them, if that

is possible on many PCs. Expensive and a real pain for system maintainers.

Changing applications means reconfiguring the PC and then rebooting.

The solution:

Suppose some small piece of software owned the Ethernet board and

provided a standard interface to all software packages. That would give

standardization for the applications, and require only one board per PC.

Further suppose this small item, named a Packet Driver, allowed

applications to register themselves by saying to the P.D. -

Pac baby:

I want packets of the following kind(s) and when you see one like

that call me at this address, immediately. (801) 555-1212.

Sincerly,

Big App

Or even the reverse if the application is packing up shop and quiting.

This second part allows multiple applications to come and go while

the tiny Packet Driver remains in memory; it uses just a few Kbytes. And it

allows several applications to register for different kinds of packets. Each

application remains ignorant of traffic not destined for itself.

Applications have to be built to use the Packet Driver interface

rather than wanting an Ethernet board to themselves. They are now being

written that way.

Packet Drivers, made simple                     2-3

Ethernet packet construction

a packet, 64 to ~1000 bytes

---------------------------------------------------------------

| preamble.. | from address | to address | TYPE |   ...data.... |

---------------------------------------------------------------

6 bytes       6 bytes     2 bytes   many bytes

Regular Ethernet uses the TYPE field to hold a number which is the

kind of protocol. Novell's TYPE is 8137.

IEEE 802.3 Ethernet (much less popular) uses that field to count

the number of bytes that follow, even though the Ethernet board itself

provides that information. Thus, software must look in the data field to

locate any protocol kind, and that will cost time and impose a load on

machines.

The TO address refers to an Ethernet hardware address, normally that

set into the board by the manufacturer and hence unique in the world. But, a

big headache is that address can be one to be received by several places or

by all places (Multicast or Broadcast). Broadcast traffic hits every single

board in the place. TCP and NFS and Unix systems love to broadcast, a lot.

Packet Driver concept, from FTP Software Inc.

--------------------    /======= coax ========>

|  Ethernet board   |- T

--------------------    \======= coax ========>

|

-----------------------------------------------------------------

Packet Driver == Owner of the Board

Board interrupts Packet Driver, saying "Got another pkt for you".

Get TYPE from board's packet buffer.

Scan list of requestors to find match of TYPE and requestor's Type.

If a match pass packet to requestor, clears board for next packet.

If no match discard packet, clears board for next packet.

------------------------------------------------------------------

|                 |              |      ... |

NetWare            TCP/IP          NFS     other requestors

Advantages:

1) Run multiple applications across the same board, simultaneously if req'd.

Each registers and can deregister itself with the Packet Driver.

2) One board fits all, no buying different boards for different applications.

Current boards: WD8003E, 3C501, 3C503, 3C523, NI5010, NI5210, DLINK,

(NE-1000 running at USU), and more directly from vendors. Recommend the

WD8003E, then the 3C503. The NE-1000 is similar to both.

3) Much faster filtering of "junk" packets, particularly and especially

Broadcast and Multicast packets found on heavily used networks.

4) Consequence of 3) the PC uses far fewer cpu cycles servicing the network.

5) No more reconfiguring and rebooting to change applications.

6) Connect to a Novell File Server (or servers) and still run TCP/IP or

PC-NFS or ... with the Novell systems remaining active and available for

file serving and printing.

7) Thus the Packet Driver acts as a fast smart secretary, bothering clients

only when packets arrive specifically for them. If an application has

slow or weak code for handling packets then the P.D. is a boon.

9) Packet Driver size is very small and the P.D. + Novell Shell (IPX + NETx)

is very nearly the same size as without a Packet Driver.

10) TYPE 8137 is REQUIRED by NetWare/VMS, no way around it.

Packet Drivers, made simple                     3-3

Other comments and opinions:

11) Alas, my experience is that ordinary Novell 2.1x File Servers are unable

to cohabit a heavily used campus backbone for some kinds of boards. But

converting the server to use Regular Ethernet packets with TYPE 8137

allows them to very rapidly discard unwanted pkts and live happily for

weeks on end attached to a main backbone. Whew!

12) BYU and Novell have made life easy for us by providing the ECONFIG.EXE

utility to convert file servers to TYPE 8137 and even to change shells

to the same system. It's reversable too. Apply to NET$OS.EX1, and to

IPX.COM (but only if not using Packet Drivers, else build IPX for PD).

13) FTP Software Inc has made life easy for us by providing the Packet Driver

specification and making it public.

14) Ethernet board vendors are rapidly supplying Packet Drivers for their

products. Hot stuff this year. From the user's side all P.D.'s are

supposed to look alike; things are improving in this regard.

15) New Packet Drivers are being developed by individuals and are available

at no cost across the networks. The central distribution point is

Clarkson University, Russ Nelson, nelson@clutx.clarkson.edu.

16) Code and prebuilt drivers are available via anonymous ftp from Clarkson

at sun.soe.clarkson.edu, 128.153.12.3,

directories pub\packet-drivers and/or pub\ka9q,

files drivers.arc  (prebuilt), and/or

driverss.arc (sources)

pcnfs.arc.Z  (PC-NFS Packet Driver support)

directory pub\novell (material from BYU)

file  novell.exe

or directly from BYU as anonymous ftp to dcsprod.byu.edu, 128.187.7.3

BYU's file NOVELL.EXE  when run unpacks to -

ECONFIG  EXE    24269   4-15-88   2:25p

PDLAN    LAN      389   4-28-89  11:48a

DPDLAN   OBJ     2266   4-27-89   6:01p

APDLAN   OBJ     2266   4-27-89   6:00p

READ     ME     10008   4-29-89   2:59p

PDSHELL  LAN      144   4-28-89  11:40a

BPDLAN   OBJ     2266   4-27-89   6:01p

PDSHELL  OBJ     1632   4-27-89   5:53p

CPDLAN   OBJ     2266   4-27-89   6:01p

where

PDSHELL.OBJ    files for 2.1 shell generation

PDSHELL.LAN         "  and go into LAN_DRV_.00n

APDLAN, BPDLAN, CPDLAN, DPDLAN, PDLAN.OBJ are

Novell 2.1 LAN driver for bridges

ECONFIG.EXE     Novell's configuration utility for

shells and drivers. (Type ECONFIG ? for help).

READ.ME         BYU's installation instructions (easy)