*************************************************************************

*                               NOVELL.DOC                              *

*                                                                       *

*      Installing and configuring the Intel TokenExpress(tm) driver     *

*                        for Novell NetWare V2.x                        *

*                                                                       *

*************************************************************************

This file contains information specific to installation of Intel's

TokenExpress adapter drivers.  When you install the TokenExpress drivers,

use your NetWare documentation for overall installation instructions, and use

this file for Intel-specific steps and parameters.

This DOC file applies to Novell NetWare versions 2.1x, 2.2 and higher

in workstations and servers.  (For NetWare 386 instructions, read the

\NOVLODLI\NOV386.DOC file.)

-------------------------------------------------------

Installing the TokenExpress drivers under NetWare V2.1x

-------------------------------------------------------

If NetWare is already installed on the hard disk or network drive,

run the NOVLINST.BAT file from the Intel diskette containing the NetWare

drivers before you generate the server boot file or IPX for a workstation.

To run the batch file, enter the following at the DOS prompt:

NOVLINST <target_path>

where <target_path>  is the path to the  directory where NETWARE is installed

For example, "NOVLINST F:\NETWARE" will create a LAN_DRV_.410 directory under

F:\NETWARE copy the  Intel  drivers  into  the  new  directory,  and  copy  a

configuration file into the AUXGEN directory (which must exist).

If you're installing NetWare for the first time on a disk/network drive, you

don't need the NOVLINST.BAT file.  During installation, NetWare will ask if

you want to copy any additional diskettes.  Enter Y (yes) and insert the Intel

diskette containing the NetWare drivers.  Follow the instructions on

screen.

If your network uses source routing, choose a configuration that enables

source routing.

----------------------------------------------------

Installing the TokenExpress drivers with NetWare 2.2

----------------------------------------------------

During the NetWare installation process, NetWare presents a list

of drivers for you to install.

1.  Press INSERT to choose a new driver.

2.  When prompted for the LAN_DRV_XXX diskette, insert the Intel

diskette containing the NetWare drivers.  Press Enter.

3.  Choose the Intel driver name from the list.

For example,   "Intel TokenExpress ISA 16/4 adapter"

Continue as directed by NetWare.

If your network uses source routing, choose a configuration that enables

source routing.

NOTE:

EARLY TOKEN RELEASE is always enabled in the Intel TokenExpress

NetWare drivers.

-----------------------------------------------

Using the NetWare drivers with IBM applications

-----------------------------------------------

If you need concurrent communication using protocols other than IPX, you must

load  the Intel DLC driver.  This  driver replaces DXMC0MOD.SYS used with IBM

Token-Ring boards.   You must then load an  IPX.COM generated for the IBM Lan

Support  Program.   This approach  gives lower  throughput and  higher memory

consumption.   The Intel DLC driver, OC02DLC.SYS, is  one of the files on the

Intel diskette containing the OS/2 EE and PC LAN drivers.

-------------------------------------------

Configuring NetWare 2.1x server packet size

-------------------------------------------

A utility OCNW21SZ.COM has been  supplied  on  the  diskette  containing  the

NetWare drivers.   It lets you  configure the packet size  for a NetWare 2.1x

server to 1k or 2k bytes.    The  utility  modifies  the  xOC02N16.OBJ  (x  =

A,B,C,D)  files.  Copy  the executable into  some directory in  your PATH and

invoke  it without arguments.  OCNW21SZ will display help information on what

arguments are needed and what the utility will do.

This utility is supplied because some 2.1x server configurations have too few

File  Service Proceesses (FSPs).   (The number  of FSPs can  be determined by

issuing the  command 'CONFIG' at  the server).   A 2.1x server  usually has 3

FSPs  when using a packet size of 2K  bytes.  This gives good parellism while

providing  high single  station throughput.   If,  because of  other hardware

installed,  less than 3 FSP's are available  (particularly if only one FSP is

available), it  may be advisable  to reduce the  packet size to  1K using the

utility.   The utility should  not be used  with other drivers.   NetWare 2.2

servers usually  have ample  FSPs.   Reducing the  packet size  will decrease

throughput with these servers.

--------------------------------------------------------------------

Hardware configurations for NetWare 2.x servers and IPX workstations

--------------------------------------------------------------------

0:     I/O = A20,A30 selfconfiguring IRQ and DMA. No Src Routing

1:     I/O = A24,A40 selfconfiguring IRQ and DMA. No Src Routing

2:     I/O = A50,A60 selfconfiguring IRQ and DMA. No Src Routing

3:     I/O = A54,A70 selfconfiguring IRQ and DMA. No Src Routing

4:     I/O = A20,A30 selfconfiguring IRQ and DMA. Src Routing

5:     I/O = A24,A40 selfconfiguring IRQ and DMA. Src Routing

6:     I/O = A50,A60 selfconfiguring IRQ and DMA. Src Routing

7:     I/O = A54,A70 selfconfiguring IRQ and DMA. Src Routing