1.  3.12 CLIB

----------------------------------------------------------------------

3.12d  10/22/93

----------------------------------------------------------------------

304.	Added GetServerUtilization and

GetServerMemorySize functions.

305.	Added Setable Parameter

debugging interface.

306.	Fixed a bug in the cleanup of threads blocked on synchronization

waits where the connection was not being reenabled.

307.	Added a ScheduleWorkToDo interface to BeginThread so that

developers could use worker threads on 4.x and have the same binary

work on 3.x

308.	Changed memory checking so it can be

used in all versions of the 3.x clib.

310.	Added code to dynamically import TotalKnownMemorysize

because it is not available in the MSEngine.

311.	Made the renegotiating of the packet size within

NCPReadFile and NCPWriteFile only happen once. This is because the

second time Netware on VMS failed.

312.	Ever since 3.1 we have been returning the ser. # and app

# swapped. Made it return correct values.

313.	Increased struct RequestPacketStructure.RData size to 539

bytes.  Also fixed the initialization of that field.

314.	Made changes mostly bracketed with __cplusplus to these headers to

clear up compilation errors or warnings under C++.

315.	OpenEA(), swapped MapPathToDirectoryNumber() for

GetEntryFromPathStringBAse() so EAs will really work with non DOS name

space paths.

316.	Added #pragma pack(); to the end of each header to return to default

packing.

317.	Fixed Daylight Savings Time check in time() to give October 31 days

instead of 30.  In 1993, it calculated that DST ended on the 24th

instead of the 31st.

318.	Added a fix to SemTimerHandler to make sure that the

process wouldn't be freed up from a semaphor if it had been signaled

right before the timer interrupt fired.

319.    Added InvertWord to GetPathFromDirectoryEntry directoryEntry

parameter.

320. 	When on an IO engine after logging in as supervisor for

connection 0, we were leaving the current directory handle in the login

directory. This didn't work very well.

321.	added DOES_NOT_RUN_ON_IOENGINE.

322.	added prototype and definition of FEMapConnsHandleToVolAndDir()

needed for 3x FSHooks.

323.	added a File System Monitor Hook Signature.

324.	added an FSHook msg. and a INIT msg.

325.	added support for FS Hooks initialization and tear down.

326.	Put SuspendOrStop in NCPRequest because a thread could be blocked

on the semaphore when it is unloaded. This will cause an abend.

327.	Added code to ExitCritSec to redistribute the link list

of TCS's so that the first thread on the link list doesn't get favored

if the NLM encounters a bunch of critical sections.

328.	Added ThreadSwitchLowPriority for backwards compatibility.

329.	made DOS, OS/2, NT, & FTAM be the default case for the

switch statements found in _NWStoreAsComponentPath() and

_NWGetDirBaseFrom Path().

330.	Added support for spaces when parsing OS2 names.

331.	A connection at unload time was being destroyed while another

thread was waiting on a semaphore. This causes an abend.

332.	added version checking for remote servers as 4.0 & 4.01

sent back serial nums. swapped.  See change 323 also.

----------------------------------------------------------------------

3.12f  12/16/93

----------------------------------------------------------------------

333.	Changed ClearConnectionNumber to not return an error when

you clear you own connection number.

334.	Changed ClearRLocks to pass FALSE as the last parameter.

335.	There was a bug found using Install where packet burst would

retry when not needed. There was also a random bug of data corruption

of 1, 2, or 3 bytes on these retries.

336.	added version checks to be able to run on

3.11 as well as 3.12.

337.	Added a check to _HandleCtrlChars to see if the OS

to clib struct ptr is null before we dereference it.

338.	Added an error code and added dynamic import

of GenNS calls so we could load in I/O engine of SFT III.

----------------------------------------------------------------------

3.12f  2/03/94

----------------------------------------------------------------------

2.  4.01 CLIB

---------------------------------------------------------------------

47.	In _DefaultParsePath() removed the assigning of 0 to *direct-

oryNumberp if ':' is found in tempPath.  this was making us blow up

later if many chdir()s were called iteratively because an

NCPFreeDirHandle was tried with dir handle of 0.

48.	In GetDataStreamName() enlarged buffer from 45 to 255 in case

OS or translations change the length of the buffer we get back.

49.	Added code to LoadLanguageMessageTable to check the message

table ptr and return an error if it is null.

50.	Changed ClearConnectionNumber to not return an error when

you clear you own connection number.

51.	Changed ClearRLocks to pass FALSE as the last parameter.

52.	On 2.X servers the DSOffset of 8000 was not working because

they would return the high byte set. Changed it to 10000.

53.	Made sure all structures and unions had tags.  Note that we DIDN'T

apply tags to structures and unions whose definition was within

another structure or union.

54.	GetLANAddress was not working remotely because it would

check for CURRENT_CONNECTIONID. This now works.

55.	There was a bug found using Install where packet burst would

retry when not needed. There was also a random bug of data corruption

of 1, 2, or 3 bytes on these retries.

56.	removed NWLstrchr() from NCPOpenDirectory() and NCPDirect-

orySearch() because the '.' we were looking form is never there since

any wild cards have already been augmented (2E -> AE, 2A -> AA, etc)

by the default path parser in opendir().

57.	Added a check to _HandleCtrlChars to see if the OS

to clib struct ptr is null before we dereference it.

3.  4.01 DSAPI

-----------------------------------------------------------------------------

80.	Fixed NWDSGetContext(), switch case DCK_NAME_CONTEXT, to write

0x0000 to the pointer passed when Unicode-to-character translation is

not set in the 'DCKFlags' instead of only '\0.'

81.	Fixed yet another logic error causing execution erroneously to

call NWDSDuplicateContext() instead of NWDSCreateContext().

82.	Fixed memory leak in NWDSDuplicateContext(). Left debugging

statements in--accessed by defining 'DEBUG' during compilation.

83.	Adjusted _NDSCreateRDNList() to return with nil by-product when

a null or zero-length object name is passed.

84.	Fixed a "Read from a nonpresent page" error in NWDSDuplicateContext().

85.	Checked 'userID' before calling NWDSDeleteUser().

86.	Added new function, NWDSListByClassAndName() and made changes necessary

to the parameter list for BuildAndSendListMessage() and to

ListAndRead() where it is called.

87.	Checked for nil- or zero-length input string to be abbreviated

and return a zero-length abbreviation. This was done in functions

AbbreviateUnicodeName() and RemoveTypesWithoutAbbrev().

88.	Modified _NDSFreeRDNList() to validate the list-field pointer

before attempting to free it.

89.	Made changes incidental to porting these to Platform-Independent

NetWare.

90.	Added validate-null mention to third argument to

NWDSAuthenticate() to enable calls to pass the RPC cross-domain

validation of pointers.

91.	Revamped for PIN port, removing Macintosh and other hardware

support to avoid confusing the issue.

93.	Added definitions for Buf_T and validated all pointers of

that type being passed in the entire library.

94.	Initialized field 'lastCount' of the newly allocated Buf_T

structure to ensure good handling through validation.

95.	Modified VCS rules paths to point to new version directory

location.

Important fix to NWDSDeleteUser() in CLib:USER.C which traverses

the connection table releasing every connection owned by the

logging-out user.

96.	Changed all the macro names from mixed case to upper case.

98.	Fixed error sending NWDSDeleteUser() a null (this had been

fixed in DSLOGIN.C months ago.

Modified code to skip DSFreeTDSData() in the case where GetTDS()

returns an error.

104.	Made changes to reintroduce the necessary pointer coercion to pass

HP's compiler. Removed occurrences of '//.'

105.	Initialized 'tempName' in the case where an error had occurred

possibly filling the string with garbage in function

GetAbbreviatedAndTypelessName.

106.	Checked second pointer to unicpy() in NWDSDuplicateContext()

to ensure nonnil. Assigned 0x0000 to 0th subscript of first argument

in failing case.

107.	Added FTOK_RDN, FTOK_BASECLS FBIT_RDN, and FBIT_BASECLS per client.

Added the latter two to FBIT_OPERAND and changed type of field

'expect' from 'uint16' to 'uint32' in filter cursor structure.

109.	Added prototype for _SameCIString().

110.	Added a new function NWDSResolveName() and a

prototype for it. Added import and RPC descriptions for this function

and NWDSAuditGetObjectID().

112.	Added new function NWDSReadSyntaxDef() and a prototype for it.

Added import and RPC descriptions for it.

113.	Removed blocks of code calling uniicmp() and _NDSApplyDefault-

NameContextRule() which were fixes the client once inserted in

NWDSCreateContext() and NWDSSetContext() but which it now no longer

desires.

114.	Fixed a few pointer-incompatibility warnings.

115.	Added change as implemented by client to BuildAndSendListMessage

which affected reading object information (I believe).

116.	Made change to parameter list of NWLocalToUnicode() and

NWUnicodeToLocal() because Jim Nicolet added an argument in Unicode.NLM

117.	Integrated changes to fix search problem for base classes. Made one

coercion myself to his fixes to pass HP's compiler.

118.	Incorporated changes in	response to the typeless/typeful naming

standardization championed by ECO recently. This holds that, by default,

names passed back from DSAPI (and DS) will be typeful since originally

they were that way. The names can be typeless if desired by calling

NWDSSetContext() and setting DCV_TYPELESS_NAMES in the context flags

(thus, NWDSCreateContext() was modified to reflect this default). Be

it noted, however, that typeless names may be passed freely to DSAPI

(and DS) whatever the state of the flag is since DS understands them

(since v4.01). passing a typeless name in a typeful (default) context

will not sollicit a typeless return however.

Important fix to NWDSDeleteUser() in CLib:USER.C which traverses

the connection table releasing every connection owned by the

logging-out user.

119.	Implemented a fix in CanonicalizeUnicodeName() where

an empty string is allowable (but still not a null pointer).

120.	Fix for trailing dots in the name.

121.	Do the internal authentication following the global resolution of the

name.

122.	Moved DCK_LAST_SERVER_ADDRESS, DCK_LAST_ADDRESS_USED

and DCK_LAST_CONNECTION from second into first. Also, provided new type

to be used in conjunction with DCK_LAST_SERVER_ADDRESS to

NWDSGetContext().

123.	Fix added to InternalAuthenticate() so that it passed context as the

first argument (unlike the client version).  In DSAPIRPC.DEF, in the

definition for Buf_T, the pointer validation attributes were changed

to DONT_VALIDATE.  This is because this strucuture may be handed to a

function uninitialized (where it is output data from NDS) or

initialized (where it is input to NDS and would therefore have valid

pointers). Since it would mean a great deal of investigation to

identify reliably the cases where it was one or the other, splitting

out a different Buf_T type to solve the ambiguity was declined and no

Buf_T fields will be validated, only the Buf_T parameter itself.

124.	References to the define DSI_DEREFERENCE_BASE_CLASS should not be made

in 4.01.

125.	Updated dsapi.imp to export both NWDSPartitionReceiveAllUpdates and

NWDSPartitionSendAllUpdates Updated dsapirpc.def to define

NWDSPartitionReceiveAllUpdates (it had already defined

NWDSPartitionSendAllUpdates).

126.	Added NWLocalToUnicode and NWUnicodeToLocal to import and RPC files.