STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 386 3.12 and 4.0 Server driver

For EtherCard PLUS LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

INSTALLATION PROCEDURE

----------------------

The purpose of this document is to briefly explain the

installation of the NetWare 386 LAN drivers that are used

with the Standard Microsystems EtherCard PLUS Family LAN adapters.

The files in the release diskette are:

INSTALL.DOC     This file.

RELEASE.DOC     Version number and specific release

information.

SMC8000.LAN     NetWare 386 3.12 Server driver.

1.  Refer to the NetWare 386 Installation manual and follow

the instructions to partition the hard disk and to copy

the necessary system files to drive C:.

3.  Copy SMC8000.LAN from the SuperDisk diskette to the

NETW_386\312 subdirectory.

4.  Follow the "Run SERVER" and the "Load loadable modules"

sections of Novell's Installation Manual to Step (3) -

Load LAN driver module.

load c:\netw_386\312\smc8000 <Enter>

5.  Answer the prompts for the hardware attributes of each

adapter. Depending on the type of adapter you have, there

are up to four adapter specific configuration parameters

the driver may ask you to supply:

1) I/O Base Address.

2) Interrupt Request (IRQ) Level.

3) RAM Buffer Base Address.

4) Size of RAM Buffer - Enter 400

5) MCA Bus Slot number (Micro Channel Only).

For the EtherCard Elite adapter, only the I/O base address

is required.

For the EtherCard Elite/A adapter, only the bus slot number

is required.

If you do not want the prompts to appear, you can include

the required configuration parameters in the command line.

Refer to the SMC EtherCard User Manual under "NetWare 386

Server Driver Installation."

ERROR MESSAGES

--------------

1.      "Could not find an adapter at I/O base address xx."

The driver was unable to find a EtherCard Elite adapter

at the specified base I/O address. Run EZSETUP to configure

the board.

2.      "Could not find an adapter at slot xx."

The driver was unable to find a EtherCard Elite/A adapter

at the specified slot ID in a Micro Channel system. Run

IBM setup program to configure the adapter.

3.      Adapter hardware failure detected (self-test)."

During self-test the driver detected a problem

with the adapter hardware. Run the DIAGNOSE program to

test the adapter.

4.      "Adapter hardware failure detected (initialization)."

During initialization the driver detected a problem

with the adapter hardware. Run the DIAGNOSE program to

test the adapter.

5.      "Cannot access adapter RAM properly."

This is an internal OS error that occurs when the driver

is unable to allocate memory needed for its local data.

6.      "Other system RAM already occupies adapter RAM."

The driver cannot load because another device is occupying

its assigned RAM space. Use the EZSETUP to configure the

SMC EtherCard.

7.      "Other system ROM already occupies adapter RAM."

The driver cannot load because a ROM device (perhaps BIOS)

is occupying its assigned RAM space.

8.      "Adapter IRQ higher than 7 not valid in 8-bit slot."

The adapter IRQ is set above 7. Reset to lower IRQ level.

9.     "Adapter unable to process a multicast address."

A node address was given on the command line or the board

is configured for a node address that is reserved for

multicast address. This can be fixed by zeroing the Least

Significant Bit of the Most Significant Byte of the node

address.

10.     "Possible cable disconnect detected on LAN adapter."

Adapter cannot detect cable connection to the network. Check

the integrity of the cable connection to the adapter.

WARNING MESSAGES

----------------

1.      Unneeded Memory Address ignored

The user specified a memory address parameter when

loading the driver that was un-needed. A software specified

memory address will be used instead.

2.      Unneeded Memory Length ignored

The user specified a memory length parameter when loading

the driver that was un-needed. A software specified memory

length will be used instead.

3.      Unneeded Interrupt number ignored

The user specified a interrupt number parameter when

loading the driver that was un-needed. A software specified

interrupt number will be used instead.

EtherCard and SuperDisk are trademarks, and SMC and Standard Microsystems

are registered trademarks of Standard Microsystems Corporation. Other

trademarks mentioned herein belong to their respective companies.